# Test Plan & Results

### 1. Test Plan

| Test | Method | Expected Result |
| --- | --- | --- |
| Gantry Mechanism (Rack & Pinion) | Tested movement across X, Y, and Z axes using DC motors | Smooth and stable movement in all directions |
| Joystick + DC Motor Control | Moved joystick in all four directions | Accurate and responsive movement |
| Claw Mechanism (Servo) | Triggered claw using button | Claw opens, goes down, grips, and lifts object |

### 2. User Testing

- Users:

  - Classmates

  - first-time users

  - self

- Tasks given:

  - Insert coin to start the game

  - Move claw using joystick (all directions)

  - Pick up an object and drop it

### 3. Results

| Test | Outcome | Pass/Fail |
| --- | --- | --- |
| Gantry Mechanism (Rack & Pinion) | Worked well in theory, but in practice the DC motor weight was too high, causing failure in X and Y axis movement | fail |
| Joystick + DC Motor Control | Successfully integrated for all four directions; movement was smooth though slightly sensitive | Pass |
| Claw Mechanism (Servo) | Servo-based opening and closing worked smoothly, but grip strength was inconsistent | Pass |

### 4. Observations

- User behavior:

  - Users quickly understood how to operate the joystick

  - Movement felt smooth and intuitive

  - Interaction was engaging and easy to learn

- Pain points:

  - The gantry structure could not support the weight of the DC motors, affecting X and Y movement. The added weight of the DC motors placed excessive load on the gantry system, causing instability and preventing smooth movement along the X and Y axes. This highlighted a mismatch between the structural strength of the frame and the mechanical load it was expected to carry.

  - Claw grip was not always strong enough to securely hold objects. While the servo-controlled mechanism functioned smoothly, the claw did not consistently apply enough force to firmly grasp objects. As a result, objects were sometimes dropped during lifting or movement, affecting the reliability of the system.

### 6. Improvements Suggested

- Gantry Mechanism: Replace heavy DC motors with lighter alternatives (like stepper motors) or redesign the structure to better support weight and improve stability.

- Claw Grip: Improve grip by redesigning the claw shape, adding rubber tips, or increasing servo torque for a stronger hold.

- Motor Efficiency: Optimize motor speed and timing for smoother and more precise movement.

- Overall Stability: Strengthen the frame and balance weight distribution to ensure consistent performance across all axes.